Excel CPU Instruction Set Manual

General Notes:

- 8-bit registers: AX, BX, CX, DX

- All arithmetic wraps modulo 256 (0..255).

- Flags: ZF (zero), CF (carry / borrow), SF (sign = MSB).

- PC (program counter) selects which instruction runs next.

- RAM is byte-addressable; STR writes AX into RAM[address].

Each instruction entry below includes:

Mnemonic / Opcode, Syntax (what you write into PROGRAM sheet), Behavior, Side Effects.

0. NOP (Opcode 0)

Syntax:   NOP

Behavior: Does nothing.

Side effects: No registers or flags change. PC just advances.

1. LDI (Opcode 1)

Syntax:   LDI AX,10

Meaning:  OP1 = register, OP2 = immediate value.

Behavior: Loads an immediate constant into a register.

Example:  AX = 10

Side effects: Only that register changes. Flags are NOT updated.

2. LDR (Opcode 2)

Syntax:   LDR BX,5

Meaning:  OP1 = register, OP2 = RAM address.

Behavior: Loads RAM[address] into the given register.

Example:  BX = RAM[5]

Side effects: Only that register changes. Flags are NOT updated.

3. STR (Opcode 3)

Syntax:   STR 11      (OP1 = 11, OP2 blank)

Behavior: Stores AX into RAM[11].

Side effects: RAM at that address changes (in the next cycle snapshot). AX itself does not change. Flags not updated.

4. MOV (Opcode 4)

Syntax:   MOV CX,AX   (OP1 = destination, OP2 = source)

Behavior: Copies one register into another. CX = AX in the example.

Side effects: Only destination register changes. Flags not updated.

5. ADDI (Opcode 5)

Syntax:   ADDI AX,7   (OP1 = register, OP2 = immediate)

Behavior: Adds an immediate constant to a register, modulo 256.

Example:  AX = (AX + 7) mod 256

Side effects:

- Updates ZF (1 if result == 0).

- Updates CF (1 if carry out > 255).

- Updates SF (1 if result >= 128).

Only that register changes.

6. ADDR (Opcode 6)

Syntax:   ADDR AX,BX  (OP1 = destination, OP2 = source register)

Behavior: Adds one register into another, modulo 256.

Example:  AX = (AX + BX) mod 256

Side effects:

- Updates ZF / CF / SF the same way as ADDI.

Only OP1 changes.

7. SUBI (Opcode 7)

Syntax:   SUBI BX,4   (OP1 = register, OP2 = immediate)

Behavior: Subtracts an immediate constant from a register, modulo 256.

Example:  BX = (BX - 4) mod 256

Side effects:

- ZF = 1 if result == 0.

- CF = 1 if a borrow was needed (old BX < 4).

- SF = 1 if result >= 128.

Only that register changes.

8. SUBR (Opcode 8)

Syntax:   SUBR AX,BX  (OP1 = destination, OP2 = source register)

Behavior: Subtracts one register from another, modulo 256.

Example:  AX = (AX - BX) mod 256

Side effects:

- ZF / CF / SF updated same way as SUBI.

- CF = 1 if borrow was needed (old AX < BX).

Only OP1 changes.

9. JMP (Opcode 9)

Syntax:   JMP 12   (OP1 = target address)

Behavior: PC = 12 (unconditional jump).

Side effects: PC changes. Registers and flags do not change.

10. JZ (Opcode 10)

Syntax:   JZ 20    (OP1 = target address)

Behavior: If ZF == 1, jump to address. Else continue.

Pseudocode: if ZF == 1 then PC = 20 else PC = PC+1

Side effects: Only PC may change. Registers/flags unchanged.

11. JNZ (Opcode 11)

Syntax:   JNZ 5    (OP1 = target address)

Behavior: If ZF == 0, jump to address. Else continue.

Pseudocode: if ZF == 0 then PC = 5 else PC = PC+1

Side effects: Only PC may change. Registers/flags unchanged.

12. MUL (Opcode 12)

Syntax forms:

MUL AX,2    (register * immediate)

MUL AX,BX   (register * register)

Behavior: OP1 = (OP1 * operand) mod 256. Example: AX = (AX * 2) mod 256.

Side effects: Only OP1 changes. Flags (ZF/CF/SF) are NOT updated by MUL.

13. LOOP (Opcode 13)

Syntax:   LOOP 2    (OP1 = target address)

Behavior:

1) CX = CX - 1 (mod 256)

2) If CX != 0, jump to the given address.

3) If CX == 0, continue to next instruction.

Side effects: CX is modified; PC may jump. Flags are NOT updated by LOOP.

14. AND (Opcode 14)

Syntax forms:

AND AX,15   (register AND immediate)

AND AX,BX   (register AND register)

Behavior: Bitwise AND into the first operand.

Example: AX = AX AND BX  or  AX = AX AND 15

Side effects: Only OP1 changes. Flags not updated.

15. HLT (Opcode 15)

Syntax:   HLT

Behavior: Halts the CPU. Execution stops; further rows keep the same register / RAM / flag values.

Side effects: Halts the machine state going forward. Does not itself modify registers.

16. OR (Opcode 16)

Syntax forms:

OR AX,1

OR AX,BX

Behavior: Bitwise OR into the first operand.

Example: AX = AX OR BX  or  AX = AX OR 1

Side effects: Only OP1 changes. Flags not updated.

17. XOR (Opcode 17)

Syntax forms:

XOR BX,AX

XOR BX,3

Behavior: Bitwise XOR into the first operand.

Example: BX = BX XOR AX  or  BX = BX XOR 3

Side effects: Only OP1 changes. Flags not updated.

18. DIV (Opcode 18)

Syntax forms:

DIV ,CX

DIV ,5

Behavior: Always divides AX by the operand (either a register or an immediate).

Quotient goes into BX. Remainder goes into DX.

Formally:

- BX = floor(AX / divisor)

- DX = AX mod divisor

- AX itself does NOT change.

Side effects: BX and DX are updated. AX and CX remain unchanged. Flags are not updated.

Summary / Quick Reference:

- LDI / LDR / STR / MOV   : data movement

- ADDI / ADDR / SUBI / SUBR : arithmetic with flags

- MUL / DIV                : multiply, divide (DIV writes BX and DX)

- AND / OR / XOR           : bitwise logic

- JMP / JZ / JNZ / LOOP    : control flow; LOOP uses CX as counter

- HLT                      : stop execution