Excel CPU Programs

This document contains assembly-style programs for the Excel CPU.

Each program is given as rows you can type into the PROGRAM sheet (MNEM / OP1 / OP2).

Addresses are shown in the ADDR column.

------------------------------------------------------------

1. Factorial

Goal: Read the value from RAM[10], compute n!, and store the result in RAM[11].

Notes:

- CX is loaded with the input n from RAM[10].

- AX is initialized to 1 and becomes n!.

- We handle the n=0 case by checking Zero Flag after SUBI DX,0.

- STR 11 writes AX to RAM[11].

Program listing:

ADDR  MNEM   OP1  OP2

0     LDR    CX   10

1     LDI    AX   1

2     MOV    DX   CX

3     SUBI   DX   0

4     JZ     8

5     MUL    AX   CX

6     SUBI   CX   1

7     JNZ    5

8     STR    11

9     HLT

Explanation:

- LDR CX,10        ; CX = RAM[10] (n)

- LDI AX,1         ; AX = 1 (running product)

- MOV DX,CX        ; copy n to DX

- SUBI DX,0        ; update flags based on n

- JZ 8             ; if n == 0, skip loop (0! = 1)

- MUL AX,CX        ; AX *= CX

- SUBI CX,1        ; CX--

- JNZ 5            ; loop while CX != 0

- STR 11           ; RAM[11] = AX (result)

- HLT              ; stop

------------------------------------------------------------

2. Sum 1..n using LOOP

Goal: Assume RAM[10] = n (1..10). Compute 1+2+...+n and store in RAM[11].

Notes:

- CX is initialized from RAM[10] and acts as both the loop counter and current addend.

- AX accumulates the running sum.

- LOOP automatically decrements CX and jumps back while CX != 0.

Program listing:

ADDR  MNEM   OP1  OP2

0     LDR    CX   10

1     LDI    AX   0

2     ADDR   AX   CX

3     LOOP   2

4     STR    11

5     HLT

Explanation:

- LDR CX,10        ; CX = n

- LDI AX,0         ; AX = 0 (sum)

- ADDR AX,CX       ; AX += CX

- LOOP 2           ; CX-- ; if CX != 0 jump back to ADDR AX,CX

- STR 11           ; RAM[11] = AX (result)

- HLT              ; stop

Result: After execution, RAM[11] holds n+(n-1)+...+1.

------------------------------------------------------------

3. GCD (Greatest Common Divisor) using Euclid's algorithm

Goal: Compute gcd(RAM[1], RAM[2]) and store result in RAM[3].

Definitions:

- AX = a

- CX = b

- DIV ,CX computes AX / CX and gives:

BX = quotient, DX = remainder

Loop:

while b != 0:

r = a mod b

a = b

b = r

Final answer is 'a'.

Program listing:

ADDR  MNEM   OP1  OP2

0     LDR    AX   1

1     LDR    CX   2

2     MOV    DX   CX

3     SUBI   DX   0

4     JZ     10

5     DIV         CX

6     MOV    AX   CX

7     MOV    CX   DX

8     JMP    2

9     NOP

10    STR    3

11    HLT

Explanation:

- LDR AX,1         ; AX = RAM[1] (a)

- LDR CX,2         ; CX = RAM[2] (b)

Loop start (ADDR 2):

- MOV DX,CX        ; DX = b (copy b)

- SUBI DX,0        ; sets flags based on b

- JZ 10            ; if b == 0, jump to store result

- DIV ,CX          ; BX = AX / CX ; DX = AX % CX ; AX unchanged

- MOV AX,CX        ; a = b

- MOV CX,DX        ; b = remainder

- JMP 2            ; repeat

Exit path:

- STR 3            ; RAM[3] = AX (the gcd)

- HLT              ; stop

End of programs.